Extracting RSA Keys and PINs from the four-channel data (C1, C3, C7, Trigger) is a systematic process that combines protocol analysis and side-channel analysis.

Here is a complete, step-by-step methodology.

### Executive Summary: The Two-Stage Attack

| Stage | Target | Primary Method | Data Source |
| --- | --- | --- | --- |
| 1. Key Extraction | RSA Private Key (d) | Power Analysis (SPA/CPA) | C1 (Power) & C3 (Clock) |
| 2. PIN Decryption | User PIN | Mathematical Decryption | C7 (I/O) & Recovered Key |

### Complete Attack Flowchart

### Stage 1: Extracting the RSA Private Key (d)

The private key is recovered by analyzing the power consumption (C1) during the RSA decryption operation.

#### Method 1A: Simple Power Analysis (SPA) - The Visual Method

When it works: When the card uses a naive square-and-multiply algorithm without countermeasures.

- Identify the Computation Window:

  - Use the C7 trigger to find the start of the VERIFY command.

  - The RSA computation typically begins 50-200 µs after the command is received.

- Analyze the C1 Power Trace:

  - Look for a repeating pattern of two distinct operations:

    - Square (S): A single, consistent power signature.

    - Square-and-Multiply (S&M): A "Square" pattern followed immediately by a different "Multiply" pattern.

- Bit Assignment:

Observed Pattern:  S   S&M   S   S&M   S   S   S&M
Key Bit (d):       0    1    0    1    0   0    1

  - This sequence directly reveals bits of the private exponent d.

#### Method 1B: Correlation Power Analysis (CPA) - The Robust Method

When it works: Always. This is the primary method for noisy traces or protected implementations.

- Data Collection:

  - Capture 200-1000 traces of different VERIFY commands with different encrypted PINs.

  - For each trace, record:

    - C1 Power Trace

    - Ciphertext (c) from the C7 data

- Attack Theory:

  - The power consumption during multiplication is proportional to the Hamming Weight of the data being processed.

  - For each bit of d, the algorithm decides whether to perform a multiplication based on that bit's value.

  - We correlate the actual power consumption with a hypothesis about each key bit.

- Implementation:

  - Use a framework like DPAlib.

  - The attack tests hypotheses about chunks of the key (e.g., 8-32 bits at a time) by predicting intermediate values in the modular exponentiation and checking which hypothesis best correlates with the power measurements across all traces.

- Output: The full private exponent d.

### Stage 2: Obtaining the Public Modulus (n)

You need both d and n to decrypt. While n can sometimes be recovered during the CPA attack, the most reliable method is to extract it from the card's certificate.

- Extract from C7 Communication:

  - Before the VERIFY command occurs, the terminal performs a READ RECORD command.

  - The response contains the ICC Public Key Certificate (Tag 9F46).

  - Parse this certificate from the C7 data stream.

- Decode the Certificate:

  - The certificate is signed by the issuer, but the public modulus n is in the clear (not encrypted, only signed).

  - Use standard ASN.1 parsing tools to extract n from the certificate data.

### Stage 3: Decrypting the PIN

With d and n recovered, you can now decrypt any captured PIN.

- Capture the Encrypted PIN Block:

  - From the C7 data, extract the encrypted PIN block from the VERIFY command data field.

- Perform RSA Decryption:

  - Compute: m = c^d mod n

  - Where:

    - c = the encrypted PIN block (from C7)

    - d = recovered private key (from Stage 1)

    - n = public modulus (from Stage 2)

- Parse the Decrypted PIN:

  - The decrypted plaintext m follows ISO 9564 Format 0 or Format 1.

  - Format 0 Example: 0x24 0x12 0x34 0xFF 0xFF 0xFF 0xFF 0xFF

    - First nibble (2) is a control field.

    - Remaining nibbles (4, 1, 2, 3, 4) form the PIN 1234.

    - 0xF padding fills the block.

Pythonic Pseudocode:

# After recovering d and n
from Crypto.Util.number import long_to_bytes

def decrypt_pin(encrypted_pin_block, d, n):
    # Convert encrypted PIN to integer
    c = int.from_bytes(encrypted_pin_block, 'big')
    
    # RSA decryption
    m = pow(c, d, n)  # m = c^d mod n
    
    # Convert to bytes
    pin_plaintext = long_to_bytes(m)
    
    # Parse ISO 9564 format
    pin = parse_iso9564_pin(pin_plaintext)
    return pin  # e.g., "1234"

### Practical Analysis Steps with 4-Channel Data

- Synchronize All Channels:

  - Use C3 (clock) to align C1 (power) and C7 (data) traces sample-perfectly.

- Parse C7 for Protocol Intelligence:

  - Trigger Point: Find the VERIFY command (INS=0x20).

  - Ciphertext Extraction: Extract the encrypted PIN block from the command data.

  - Modulus Extraction: Find READ RECORD responses earlier in the trace that contain the ICC PK Certificate (tag 9F46).

- Analyze C1 for Key Recovery:

  - Segment: Isolate the portion of the C1 trace that corresponds to RSA computation.

  - SPA: Look for obvious square-and-multiply patterns.

  - CPA: If SPA is inconclusive, use the collected ciphertexts and power traces to perform a correlation attack.

- Validate and Decrypt:

  - Use recovered d and extracted n to decrypt a known PIN block.

  - Verify the output is a valid PIN format.

### Summary of Data Usage

| Channel | Role in RSA/PIN Attack |
| --- | --- |
| C1 (VCC) | Primary Key Extraction: Power consumption analysis reveals private key bits through SPA/CPA. |
| C3 (CLK) | Synchronization: Provides precise timing to align power traces with protocol events. |
| C7 (I/O) | Context & Data: Provides the encrypted PIN, trigger point, and public modulus from certificates. |
| Trigger | Capture Initiation: Ensures the data acquisition starts at the correct moment. |

This methodology shows that with high-quality 4-channel data, recovering RSA keys and PINs becomes a systematic process of combining protocol information with side-channel signals.