Obtaining Track 2 Equivalent Data and 3DES keys from the four-channel data (C1, C3, C7, Trigger) can be done through a combination of direct protocol analysis and side-channel analysis.

Here is a practical guide focusing on simpler, more direct methods before resorting to complex statistical attacks.

### Executive Summary: Two-Pronged Approach

| Target | Primary Method | Secondary (SCA) Method |
| --- | --- | --- |
| Track 2 Data | Direct Parsing from C7 (I/O) | Power/EM analysis on memory access |
| 3DES Keys | Power Analysis (SPA/CPA) on GENERATE AC | Template Attacks / DPA |

### Step 1: Obtaining Track 2 Equivalent Data

The goal is to get the data used to create the Application Cryptogram (ARQC/TC).

#### Method 1A: Direct Extraction from C7 (I/O Line) - SIMPLEST

The most straightforward method is to passively decode all communication on the C7 line.

- Capture the Transaction Flow:

  - Use your logic analyzer or decoded C7 data to record the entire transaction sequence.

  - The key APDU commands to look for are:

    - SELECT (AID)

    - GET PROCESSING OPTIONS (GPO)

    - READ RECORD (Multiple)

    - GENERATE AC

- Extract Data Elements from READ RECORD Responses:

  - The GENERATE AC command uses data that has been previously read from the card.

  - By parsing the READ RECORD responses, you can extract the critical, static data that forms the input to the cryptogram:

    - PAN (Primary Account Number - Tag 5A)

    - Card Expiry Date (Tag 5F24)

    - Service Code (Tag 5F30)

    - Application Interchange Profile (Tag 82)

    - Card Verification Results (CVR) (Tag 9F10)

  - This collection of data is the functional equivalent of Track 2.

- Assemble the "Track 2 Equivalent" Input for GENERATE AC:

  - The GENERATE AC command payload itself contains dynamic data:

    - Unpredictable Number (from the terminal, Tag 9F37)

    - Amount, Authorized (Tag 9F02)

    - Terminal Verification Results (TVR) (Tag 95)

  - The complete input to the 3DES algorithm is a structured concatenation of these static and dynamic data elements (as defined in EMV spec, Book 2).

Why this works: You are not breaking any cryptography. You are simply eavesdropping on the normal, unencrypted communication between the card and terminal to collect all the necessary data elements.

### Step 2: Obtaining the 3DES Keys

This is where side-channel analysis of the C1 (Power) and C3 (Clock) signals becomes essential.

#### Method 2A: Simple Power Analysis (SPA) on the C1 Trace - FOR VISUAL CLUES

This is a good first step to understand the card's operation.

- Trigger and Capture:

  - Use the GENERATE AC command (INS=0xAE) on C7 as your trigger.

  - Capture a high-resolution power trace (C1) synchronized with the clock (C3).

- Visual Identification of 3DES Rounds:

  - 3DES involves 48 DES rounds (16 rounds x 3 keys).

  - In the power trace, look for a repeating "bushy" pattern that cycles 16 times, followed by another 16 slightly different rounds (for the second key), and so on.

  - What you learn: This confirms the algorithm and helps you identify the precise time window where the first and last rounds of 3DES occur. This is crucial for setting up the more advanced CPA attack.

#### Method 2B: Correlation Power Analysis (CPA) - THE PRIMARY METHOD

This is the most effective and direct key-recovery method. It correlates the power consumption with hypotheses about the key.

Concept: The power consumed when a specific S-Box processes data is correlated to the number of bits that change (Hamming Weight/Distance). By testing all possible subkeys for an S-Box and seeing which one correlates best with the actual power trace, you recover the key piece-by-piece.

Step-by-Step CPA for 3DES:

- Capture a Large Dataset:

  - Perform hundreds to thousands of GENERATE AC transactions.

  - For each transaction, ensure the Unpredictable Number (Tag 9F37) changes. This provides the varying input data needed for the attack.

  - For each transaction, save:

    - The power trace (from C1).

    - The input data to the 3DES algorithm (collected from C7 as in Step 1).

- Set Up the CPA Attack:

  - Use a tool like ChipWhisperer, DPAlib, or a custom Python script.

  - Target the First Round. It's the easiest to attack because its input (the plaintext) is known from the C7 data.

  - Focus on a Single S-Box. There are 8 S-Boxes in DES. You will attack them one by one.

- The Attack Loop for One S-Box:

  - For a specific point in time (sample) within the first round's power trace:

    - For all N captured traces, you know the input plaintext P[i].

    - For each candidate 6-bit subkey k (0 to 63):

      - For each trace i, calculate the 6-bit input to the S-Box.

      - Compute the hypothetical 4-bit output of the S-Box.

      - Model the hypothetical power consumption H[i, k] (e.g., using its Hamming Weight).

    - For each candidate key k, compute the Pearson correlation coefficient between the vector H[:, k] (all hypothetical power values) and the vector of actual power measurements T[:, sample] (all traces at that sample point).

# PSEUDOCODE for CPA on one S-Box
for candidate_key in range(64): # 2^6 possibilities
    hypothetical_power = []
    for i in range(num_traces):
        sbox_input = (plaintext[i] ^ candidate_key) # Simplified
        sbox_output = des_sbox_lookup(sbox_input)
        hypothetical_power.append(hamming_weight(sbox_output))
    
    # Correlate with actual power traces
    for sample in range(trace_length):
        correlation = pearsonr(hypothetical_power, all_traces[i, sample])
        if abs(correlation) > threshold:
            print(f"Found S-Box key: {candidate_key} at sample {sample}")

- Recover the Full Key:

  - The candidate key that produces the highest correlation peak across many traces is the correct 6-bit subkey for that S-Box.

  - Repeat this process for all 8 S-Boxes to recover the full 48-bit round key.

  - Derive the original 56-bit DES key from this round key.

  - Repeat the entire process for the second and third DES keys in the 3DES sequence.

### Summary: The Simple Analysis Pipeline

graph TD
    A[4-Channel Raw Data] --> B{Parse C7 I/O Line};
    B --> C[Extract Static Data<br>PAN, Expiry, AIP, CVR];
    B --> D[Capture GENERATE AC<br>with Dynamic Data];
    C & D --> E[Reconstruct Full<br>3DES Input Message];
    
    A --> F[Analyze C1 Power Trace];
    F --> G[SPA: Identify 3DES Rounds];
    G --> H[CPA: Recover 3DES Keys];
    
    E & H --> I[Validate Key:<br>Recalculate MAC];
    I --> J[Success: Track 2 Data & 3DES Keys Obtained];

By combining passive eavesdropping on the C7 data line with active side-channel analysis of the C1 power line, you can systematically recover both the Track 2 equivalent data and the 3DES keys without needing deep cryptanalysis.